- ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

# Contract title: Delivery of specialized firefighting equipment	p 1 / 12

Publication reference: CB005.2.11.035-5

Columns 1-2 should be completed by the contracting authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the contracting authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words ‘compliant’ or ‘yes’ are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item number | 2. Specifications required | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
| 1.1 | Portable fire pump Certified according EN 14466, Flow according PFPN 10-1000 / PFPN 10-1500 The pump is made of light alloy; Aluminium frame with four pivoted handles; Automatically controlled double action pump with manual cut-off; 1 х Suction hole type Storz A with coupling; 2 x Discharge holes with Storz B type couplings, shut-off valves and safety caps; Casing with floodlight, operating panel with revmeter, operation hour counter, and control LEDs for battery charge status, oil pressure, fuel reserves, engine malfunction. This portable pump will be used for transporting water over long stretches with the high maximum discharge pressure would make it possible to use fewer pumps |  |  |  |
| 1.2 | Suction hose Certified according to EN 14557 or equivalent Diameter: compatible with position 1.1 Length: min 2 m; Made of synthetic rubber, corrugated cover, steel wire reinforced; Maximum pressure loads - 6 bar, Maximum vacuum load - 0.8 bar |  |  |  |
| 1.3 | Suction strainer Certified according to DIN 14362 or equivalent. Compatible with position 1.1 and 1.2 Made of lightweight aluminium alloy |  |  |  |
| 1.4 | Pressure hose Compatible with position1.1, 1.2 and 1.3 L= min20m, made of aluminium alloy |  |  |  |
| 2 | Submersible pump type I Certified according to DIN 14425 or equivalent. The integrated flat suction device shall be able to pump out clean or dirty water, also with abrasive sand and solids up to 10mm. Fully submersible, pressure tight electric motor. Connection voltage: 400 V / 3 phases; Flow rate Q – min 1200 l/min; Storz B coupling; Max. Weight – 33 kg Motor housing, Suction strainer (Sea water resistant aluminium casting), Suction sieve, Screws, Nuts (Stainless steel), Motor Jacket (Stainless steel) Motor shaft (Stainless steel); feed-through area (cable length: min 20 m) |  |  |  |
| 3 | Submersible pump type II Certified according to DIN 14425 or equivalent. The integrated flat suction device shall be able to pump out clean or dirty water, also with abrasive sand and solids up to 8mm. Fully submersible, pressure tight electric motor. Connection voltage: 230 V / 1 phase; Flow rate Q – min 700 l/min; Storz B coupling; Max. weight – 25 kg Motorhousing, Suction strainer (Sea water resistant aluminium casting), Suction sieve, Screws, Nuts (Stainless steel), Motor Jacket (Stainless steel) Motor shaft (Stainless steel); feed-through area (cable length: min 20 m) |  |  |  |
| 4 | Nozzle Certified according to the standard for nozzles EN 15182 or equivalent, with the Storz C clutch With adjustable flow rates (min 130 l/min – max 400 l/min) at 6 bar operating pressure of the nozzle by rotating “ring” with not less than 3 working positions. Ergonomic pistol grip for better handling and reduced fatigue; Rotating teeth made of stainless steel, water carrying components corrosion-resistant materials (brass, aluminium and man-made materials); adjustable flow and jet shape adjustment from compressed jet to mist type jet |  |  |  |
| 5 | Portable Hydraulic Jack Lifting force: min 50 KN (min 5 tons); Maximum lift - not less than 250 mm; Including the obligatory accessories for normal working |  |  |  |
| 6.1 | Rescue lifting bag Certified according to EN 13731 or equivalent Lifting bag set that works up to maximum lifting power min 10 t; Insertion height: min 2.5 cm; Max lifting height: min 19 cm; Including the obligatory accessories for normal working |  |  |  |
| 6.2 | Pressure reducing valve Operation pressure 200/300 bars Compatible with position 6.1 |  |  |  |
| 6.3 | Totten controller Certified according to EN 13731 or equivalent It requires the use of a totten controller in accident rescue activities. Compatible with position 6.1 |  |  |  |
| 6.4 | Hose for inflating of the pillow Length min 5 m, operating pressure 10 or 12 bars, compatible with position 6.1 |  |  |  |
| 6.5 | Steel Air Cylinder Capacity: min 6l/ operating pressure 300 bars, compatible with position 6.1 |  |  |  |
| 7.1 | Combined hydraulic rescue tool for cutting and spreading A tool that combines a spreader and a cutter in one rescue tool, conforms partly to EN 13204; Cutting Class - min „I“; Cutting force up to min 380 kN; round steel up to min 32 mm; Spreading force min 25 mm from tips - min 35kN; Spreading distance –min 360 mm; Pulling force up to – min 48 kN. |  |  |  |
| 7.2 | Battery The battery with the longest possible operating time suitable for position 7.1; min 5Ah- Full battery power up to -20 °C; With charge indicator |  |  |  |
| 7.3 | Charging device 220V Suitable for the proposed battery in position 7.2 |  |  |  |
| 8.1 | Electric chainsaw Device with mechanical safety switch, automatic chain brake, chain-chater. Bar length: min 40 cm |  |  |  |
| 8.2 | Protection helmet It has textile harness for comfort and low pressure against the head and includes visor with clear view in low light. The hearing protectors have optimal ergonomics for prolonged use. |  |  |  |
| 9.1 | Scaling ladder Certified according to EN 1147 The ladder allows to be used by two persons with a minimum weight capacity of 216 kg. The beams are made of high-strength light alloy. Grooved strong square rungs. The ladder system is compatible with all usual ladders. |  |  |  |
| 9.2 | Bridge for scaling ladder Certified to EN 1147; It transforms 2 or 4 scaling ladders into a freestanding ladder by placing on top of the ladders (to create a bridge). It is made of reinforced, durable light alloy |  |  |  |
| 10.1 | Firе Jacket It meets the requirements of EN 469, or equivalent. Protection level Xf2, Xr2, Y2, Z2 The jacket is multi-layered and consists of: - Outer layer - is made of flame- and heat resistant min 195 g/m2 98% META-PARA-ARAMID, 2% antistatic fiber; - Intermediate layer - flame retardant breathable membrane, protecting from the penetration of liquids; - Lining - insulation of flame retardant stitched material. The jacket's shoulder seam is low-necked, giving high freedom of movement. It is closed by a highly heat resistant zipper which is then covered by a panel, High collar fully lined with throat closure tab, Set-in sleeves, 2 bellows type breast pockets; Main colour of jacket – navy blue |  |  |  |
| 10.2 | Firе Trousers They meet the requirements of EN 469, or equivalent. Protection level Xf2, Xr2, Y2, Z2 The trousers are multi-layered and consist of: The outer shell is made of tough flame- and heat resistant material with carbon fibre (which prevents electrostatic charges); The middle lining is a breathable and waterproof membrane; This membrane will also act as a vapour barrier, protecting the fireman during extinguishing operations, A thermal liner that is permanently sewn in the trousers ensure temperature insulation; The lining is made of suitable material for the human body in a high temperature or humidity; 2-side pockets, fastened waist tab to adjust fit; Reinforced in the knees. For drainage of water / against soaking /, in the lower inner part of the legs to have rubber bands; Main colour of trousers – navy blue |  |  |  |
| 10.3 | Fire-fighting helmet It meets the requirements of EN 443 type B or equivalent. It has an ergonomic, optimised 3-point chin and neck strap made of heat and flame-resistant material; The helmet has a movable visor retractable inside the helmet; The helmet is suitable for head diameter - not less than 63 cm; An inner lining provide maximum safety and wearing comfort. |  |  |  |
| 10.4 | Fireman's Safety Belt It meets the requirements of EN 358 and DIN 14927, or equivalent. It is made of min 85 mm polyester webbing, flame retardand, black, with dual thonged buckle, with safety rope min Ø 12 mm , length min 800 mm and hook. |  |  |  |
| 10.5 | Firefighting Gloves with cuff They meet the requirements of EN388 or EN 659 or equivalent. Five-finger gloves are made of leather with knuckle protectors and cuff at the wrist, which is extremely supple and remain supple and flexible even after frequent soaking and drying of glove. The gloves are waterproof and breathable. Erasure resistance - performance level not less than 3 Piercing resistance - level of execution not less than 3 Shear resistance - level of performance not less than 2 Tear resistance - level of performance not less than 3 Combustion behaviour - level of performance not less than 4 |  |  |  |
| 10.6 | Fire protective boots They meet the requirements of EN 15090 or equivalent They are made of hydrophobic waterproofed leather, lined with membrane watertight and breathable, protection against viruses, bacteria, bodily fluids and various chemicals, Ankle protection for optimum safety at operations; The shoes have antistatic outsole with air chambers that shall guarantee excellent insulation from heat and cold built-in protections against reduction of the toes and against piercing of the foot. The shoes allow easy and fast putting on and taking off, by means of a removable zipper with a cast tooth, mounted on each detail The shoes comply with type F2A The shoes meet at least type HI 2 They have reflective elements at visible areas; easy to clean |  |  |  |
| 11 | Rain protection set (Trousers, Coat with hood) It is manufactured in accordance with EN 343 or equivalent Made of a double PVC-coated nylon material with good resistance to hazardous liquids. Enormous tear resistance. High-frequency welding of the seams that shall guarantee an absolute watertightness; press stud closure, rain cahnnel, back yoke and two side pockets with flap. |  |  |  |
| 12 | Portable LED projector Handlamp approved for professional use. Light Source: LED; Beam Distance min 160 m; Rechargeable battery/ies included; LED lamp to illuminate - not less than 90 lumens; Device protection - not less than IP 54; |  |  |  |
| 13 | Lighting System The smart LED lighting system that ensures a lighting duration of work min 8 hours; 4000 lumens; it allows the user to regulate the intensity of the light, choose the desired run time; Turnable light head that allows movement in all directions; included charger 12/24 V 2 m with plug for cigarette lighter; intelligent monitoring system that prevents over- and discharge and protects own and vehicle battery; a sturdy stable base for mast at full extension. |  |  |  |
| 14 | Stretcher foldable It meets the requirements of DIN 13024 N or equivalent. It has aluminium basic frame, Hard-wearing cover made of chemical fibre fabric, Integrated head pillow pouch, Firm, lockable joints for folding, Two adjustable belt straps with quick locking device for securing of patient. Minimal storage/packing space required. |  |  |  |
| 15.1 | Basket stretcher with belts, mat and footrest Basket stretcher is made of high quality, impact resistant and hard-wearing material, with a full aluminium tube frame. The basket stretcher will be used horizontally or vertically. The basket can be used as a dragging basket or as an abseil basket stretcher. Four large metal reinforced rings for attaching karabiners need be fitted on the sides. They are corrosion, chemical and extreme temperature resistant. The side walls act as protection against pressure and collisions, it includes adjustable foot supports (depending on patient size), continuous recessed grips on the sides, continuous nylon rope which the safety belts can be attached to. |  |  |  |
| 15.2 | Lifting bridles With 4 carabineers for basket stretcher compatible with position 15.1 |  |  |  |
| 15.3 | Carrying bag Bag for basket stretcher compatible with position 15.1 |  |  |  |
| 15.4 | Six-point belt in extreme rescue situations for spine-boards and basket stretchers Special 6- point SCHROTH belt to quickly and safely secure patients in extreme rescue situations compatible with position 15.1 |  |  |  |
| 16 | Life - jacket "SAFETY AUTOMATIC" It is certified as to EN 396 or equivalent. Faint-proof life jacket. Inflation occurs upon water contact by means of a CO2 cartridge or by manual release with the toggle fastening. Outfitting: weld together air balloon with automatic-release, oral inflation tube, strong holder with snaps, variably adjustable waist belt with insert buckle, 6 reflector strips on flotation balloon, Uni size (60 - 120 kg) |  |  |  |
| 17 | EMERGENCY BACKPACK FOR FIRE BRIGADES High-grade bandage material, such as textile triangle-bandages, rescue-cover, aluminium bandage, sterile wound-compress, various bandages for fixing, mull, momentary-bandages, finger-bandages, various plasters and leather finger-stalls, etc. |  |  |  |
| 18 | WARNING PYRAMID WITH DANGER SYMBOL, 60 CM, Foldable traffic sign with lettering. With covering. |  |  |  |
| 19 | Flashing foldable traffic control cone 60 CM Visible from all directions (360°) from over 900 feet. With integrated, powerful flashing light for even better visibility in the dark or under twilight conditions. Easy handling. |  |  |  |
| 20 | Europe barrier tape The non-adhesive tape in roll 500 m long and 80 mm wide. Red/white printed stripes on both sides. |  |  |  |
| 21 | Power Generator It meets the requirements of DIN 14685 or equivalent. It is suitable for use also by Fire departments or users who require a higher level of protection, power min 8 kVA with digital display, working in emergency mode. Type – synchronized with electronic controller; Voltage – 230/400 V; 2-cylinder and 4-stroke gasoline engine, cool cleaner, Consumption – approx. 6 l/h under full load, user manual (certificate) for appropriate use in EU with CE Declaration of Conformity |  |  |  |

- The tenderer shall warrant that the supplies are new, unused, of the most recent models and incorporate all recent improvements in design and materials.

- The Technical Specification describes the minimum requirements. Participants should offer equipment fully covering or exceeding the minimum parameters and functionalities set out in this specification.

- The supplies must comply with the most recent European standards for the respective equipment.

- At each reference to a standard, specification, technical evaluation, technical approval or technical standard above, the Contracting Authority shall also accept the equivalent (s).

- For those specified in the Technical Specification: specific standard, specification, technical evaluation, technical approval, technical standard, specific process or method of manufacture, specific model, source, specific process that characterizes the product or service, trademark, patent, type, specific origin or read production to be supplemented by the words "or equivalent".