# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title : Supply of firefighters belts and personal equipment for forest fires	p 1 /17

Publication reference : CB005.1.11.005-SUP2

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| I | Supply of PERSONAL PROTECTIVE EQUIPMENT, PROTECTING THE HEAD – 36 sets with minimal technical requirements: - Protective helmet – To be in comply with the requirements of Bulgarian State Standard (BDS) EN 12492: 2012 – „Height-rescue equipment. Helmets. Safety requirements and testing methods” or equivalent; - To be ventilated; - To be applied to Butt lantern lighting; - Main color – red or orange; - To be CE marked. - Butt lantern lighting – -LED lighting; -To be able to be used (the positioning of the face of the head), with and without a helmet; -Minimum running time at the maximum to the main / s battery / s - 8:00. -Supply the batteries with rechargeable AAA; -Be complete with main and spare batteries -Battery charger for 220V and 12V -Minimum distance of illumination - 15 m; -At least three modes - constant flashing, economical, etc.; -To have CE marking. - Protective goggles -To meet the requirements of Bulgarian State Standard (BDS) EN 166: 2003 "Individual eye protection. Requirements "or equivalent - optical class 1; protection and markings - 1.V.3.4.9; - Polycarbonate, panoramic glasses with indirect ventilation – anti-sweating lenses; -anti-allergic softening seal; - Provide protection against dust, liquids and splashes of hot particles; - To have CE marking and according to EN 166 - 1.B.3.4.9 - Firefighting hood - Hood designed to protect the head and neck from high temperatures and flame; - To meet the requirements of: - Bulgarian State Standard (BDS) EN 13911: 2006 - "Protective clothing for firefighters. Requirements and test methods for hoods for firefighters” or equivalent. Warranty periods: 1. Flashlight for frontal lighting and goggles - not less than 6 months 2. In the event of defects in the warranty period, PRINCIPAL writes notice to the Contractor. The warranty period shall be suspended for the period of written notification to the removal of the defect. 3. In cases of defects in the warranty period, Contractor shall replace the damaged item with a new for 14 days. In this case, the warranty period shall be extended by the time replacement. 4. All costs associated with the removal of defects during the warranty period, including transport costs are borne by the contractor. |  |  |  |
| II | Supply of PERSONAL PROTECTIVE EQUIPMENT, PROTECTIVE BODY AND AIDS – 36 sets with minimal technical requirements: - Two-piece suit with an overlap – - The suit is designed to protect the body of firefighters in carrying out firefighting operations on open terrain (forest conditions) and related activities in areas subjected to heat and / or flame; -To meet the requirements of: - Bulgarian State Standard (BDS) EN 15614: 2007 "Protective clothing for firefighters. laboratory test methods and performance requirements of clothing for firefighting in open country" or equivalent; - Bulgarian State Standard (BDS) EN ISO 11612: 2008 "Protective clothing. Clothing for protection against heat and flame." or equivalent - Bulgarian State Standard (BDS) EN 1149-5: 2008 "Protective clothing. Electrostatic properties. Part 5: Characteristics of the material and design requirements " or equivalent; - Consist of two parts - jacket and trousers with overlap of the two parts at least 15 cm by bending, stretching or turning; - Be made of textiles certified Bulgarian State Standard (BDS) EN ISO 11612: 2008, Bulgarian State Standard (BDS) EN 1149-5: 2008 and Bulgarian State Standard (BDS) EN 15614: 2007 or equivalent, containing not less than 50% aramid fibers and surface density (specific gravity) of textiles 220 g / m² ± 5%.; - Is made with auxiliary materials; thread seams - 100% aramid, to answer to the requirements of paragraph 6.5 of Bulgarian State Standard (BDS) EN 15614: 2007 or equivalent; zip to answer the requirements of paragraphs. 6.4 by Bulgarian State Standard (BDS) EN 15614: 2007 or equivalent tapes "Velcro" with a width of 25 mm, to answer to the requirements of item. 6.4 by Bulgarian State Standard (BDS) EN 15614: 2007 or equivalent; Reflective tape width min 50 mm, to answer to the requirements of item. 6.4. by Bulgarian State Standard (BDS) EN 15614: 2007 or equivalent; -The various parts of the suit have a label permanently affixed on the inside of the garment, which remain legible after 50 cleaning cycles and contain the information required by Bulgarian State Standard (BDS) EN 15614: 2007 or equivalent; -Background color of clothing - navy. a) Trousers - The trousers have a front fastening, high waist and adjustable straps made of elastic tissue; - Belt pants in the side parts is elastic, and its front fastening be met by flat metal hook; - Fasten trousers to place a zipper; - The legs are reinforced in the knee; the pockets are: two side pockets with flaps, closed with ribbon type ''Velcro'' laterally in the area above the knee - one on each leg. The covers of the pockets are made so that it can be slipped into your pocket and be 20 mm wider than the hole; - Reflective strips are placed across the leg below the knee in accordance with Bulgarian State Standard (BDS) EN 471: 2003 + A1: 2008 "Protective clothing with high visibility for professional use. Test methods and requirements "or equivalent. b) Jacket - To open the front zip through it and overlapping the entire length cover with adhesive tape "Velcro; - Sleeves end with shtrifelka tape "Velcro" that allows adjusting the diameter of the sleeve. - Pockets: two upper pockets located on the chest, of which one radio station and two lower side pockets. All pockets are closed by lids and tape "Velcro". The covers of the pockets are made so that it can be slipped into your pocket and be 20 mm wider than the opening. - Collar to cover the neck and fastened with extended language bar "Velcro", so as not to form an opening in the throat; - On the back to be put durable reflective inscription FIRE & RESCUE, color as same as the reflective tapes, by height 80 mm and length 300 mm (shtrifelka impact, with capital letters divided to dimensions 80/300 mm); - The reflective tapes to be put: horizontally on chest and back; horizontally on the lower part of the jacket and vertically on sleeves according to Bulgarian State Standard (BDS) EN 471:2003 +А1:2008 „Protective cloth with high visibility for professional utility. Test methods and requirements or equivalent." - Vest - Have a straight silhouette with acute neck; - Main textiles to meet the requirements of Bulgarian State Standard (BDS) EN ISO 11612: 2008 - Class A1, A2, B1, C1 or equivalent; - The main textile is surface density (specific gravity) at least 220 g / m2; - Fastening is through the zipper and adhesive tape "Velcro"; - On the front are sewn two upper and two lower pockets to be closed with shutters and tape "Velcro"; - On the back of the full width in the waist to have an external pocket with a depth of 250 mm, a closable zipper at the top; - On the back to be put durable reflective inscription FIRE & RESCUE, color as same as the reflective tapes, by height 80 mm and length 300 mm; - Color of main textiles – orange; - Is lined with lining. - Protective gloves - Are: with five fingers; overall length of at least 35 cm cuff; internal knitted cuff and shtrifelka with adhesive tape for wrapping the outer cuff, sewn loop for attachment to a belt; - Material – processed leather, lined with fabric and reinforced with an additional layer by hand; - Complying of Bulgarian State Standard (BDS) EN 388: 2006 „Gloves for prevention of mechanical risks” or equivalent, Bulgarian State Standard EN 407: 2006 "Gloves for protection against thermal risks (heat and / or fire)" and Bulgarian State Standard (BDS) EN 420: 2003 + A1: 2010 "Protective gloves. General requirements and test methods "or equivalent; - To qualify for the level of cut resistance ≥ 2 according to EN 388: 2006 or equivalent; 4. Rucksack - Volume to be at least 40 liters; - Breathing back with ventilate canal; - Big front pocket; - Side by side pockets; - Reinforced bottom; - Straps adjustable fastening; - With cross strap attachment; - Close at the top of the backpack - A lid and zipper; - Material: synthetic and waterproof; - Be permanently placed contrasting inscription of the visible part - the face of the backpack, FIRE & RESCUE height 60 and 200 mm in length, font Impact; - Background color - dark blue. Warranty periods: 1. For the trousers, jacket and rucksack - not less than 24 months and gloves - not less than six months. 2. In the event of defects in the warranty period, the supplier shall notify in writing the contractor. The warranty period shall be suspended for the period of written notification to the removal of the defect. 3. If defects appear during the warranty period, Contractor shall replace the damaged item with a new in 14 days. In this case, the warranty period shall be extended by the time replacement. 4. All costs related to the elimination of defects during the warranty period, including transport costs are borne by the contractor. |  |  |  |
| III | Supply of PROTECTIVE BOOTS – 36 PAIRS with minimal technical requirements: Protective boots should comply with Bulgarian State Standard (BDS) EN ISO 20345: 2011 "Personal protective equipment. Protective boots" or equivalent. Materials: Person - Hydrophobizing leather from cattle, black, thick 1.8 -2.00 mm; Lining - nonwovens, fusing with foam, black; Lining - nonwovens; Leather, 3 mm thick; Insole –anti-bacterial, relaxing, breathable (with perforation), mobile, softened, washable at 30 ° C; Sole - mixtures of wear resistant thermo rubber tread to have resistance to sliding. The foot to withstand contact with a hot surface of up to 250 ° C for forty minutes, the protection of the foot of the stabbing - not less than 1100 N, with a protective plate; Bombay -with protection from crushing - not less than 200J. Construction: The main facial details must be with internal seam, conch, and external fillet with leather "ear", language and softened collar. The height of a horse in some spots is min. 170 mm - not more than 180 mm average number 42. Attach tops to be done with four-stitching and the fillet to the tops with double-row stitching for optimal strength. The density of stitches in the crucible is 3.0 - 3.5 stitches / cm. Conch at the top end with a padded collar. The width of the softened collar is 45 mm ÷ 48 mm at the top of a horse, the collar is broaching a crucible that forms the two softened areas. The language is a bellows structure - water - and dust-protected and consists of three parts, the middle part is completely softened. Sayat should have a system for fast and easy footwear and take off the shoe - removable zip molded tooth, black, mounted on a leather detail with additional closure at the top with velcro tape and a symmetric caps for connections on both sides of the rolling leather detail (2 x 8 pcs.) Warranty periods: 1. Protective boots - no less than 6 months 2. In the event of defects in the warranty period, PRINCIPAL written notice to the Contractor. The warranty period shall be suspended for the period of written notification to the removal of the defect. 3. If defects during the warranty period, Contractor shall replace the damaged item with a new for 14 days. In this case, the warranty period shall be extended by the time replacement. 4. All costs related to the elimination of defects during the warranty period, including transport costs are borne by the contractor. |  |  |  |
| IV | Supply of 36 firefighting belts with minimal technical requirements: General requirements Safety belt with lanyard for the purpose of height work according to Bulgarian State Standard (BDS) EN 358 or equivalent and DIN 14927:09 Accessories to the belt 1. Pick Head Fire Axe – small, in case, according to DIN 14924:01 2. Key for connectors type Storz В/С – in case, according to DIN 14 822 3. Insulating pliers – in case – in case, for operating facilities under high tension ≥1 kV 4. Firefighting hose holdback, according to DIN 14828:10 The belts are in a set with all the accessories Warranty of firefighting belt At least 12 months |  |  |  |