VOLUME 4.2

FINANCIAL OFFER TEMPLATES

LUMP SUM CONTRACTS

Introduction

1.	The Breakdown of the Lump-sum Price (Volume 4.2.3) is the itemised list of prices showing the build-up of the price in a lump-sum contract. This Breakdown of the Lump-sum Price does not derogate in any way to the clause stating that, in a lump-sum contract, the total contract price remains fixed irrespective of the quantity of work actually carried out.

The breakdown must coincide with the payment-definition chosen in article 49 of the Special Conditions: by the tranches specified in article 49(1) of the Special Conditions.

2.	The item description given in the Breakdown of the Lump-sum Price in no way limits the Contractor’s obligations under the Contract to provide all the works described elsewhere.

3.	The prices of the Breakdown of the Lump-sum Price include all incidental and contingent expenses and all risks necessary to construct, complete and maintain all works in accordance with the Contract. Unless separate items are provided in the Breakdown of the Lump-sum Price, prices include all costs involved in the various items of the Breakdown.

4.	The prices do not include taxes and fiscal duties, as exoneration is explicitly given for the Contract. Non-exonerated taxes and fiscal duties are, apart from those stated separately in the financial offer templates, covered in the Lump–sum Price of the Contract.

VOLUME 4.2.3 — BREAKDOWN OF THE LUMP-SUM PRICE

| Item | Description | Unit | Unit price [EUR] | Firm Quantities | Lump-sum price [EUR] |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| I | PART: ARCHITECTURE |  |  |  |  |
|  | DISASSEMBLY WORKS |  |  |  |  |
| 1 | Disassembly of the metal door in the middle of the bridge | piece |  | 1.00 |  |
| 2 | Disassembly and transport of the metal railing | m |  | 132.00 |  |
| 3 | Disassembly of existing light pillars | piece |  | 6.00 |  |
|  | CONSTRUCTION WORKS |  |  |  |  |
| 4 | Mastic on the direct axis of the bridge's board | m3 |  | 2.01 |  |
| 5 | Facing on the direct axis of the bridge's board | m |  | 139.30 |  |
| 6 | Facing on the bridge's pillars | m2 |  | 203.00 |  |
| 7 | Mastic on the bridge's pillars | m3 |  | 3.45 |  |
| 8 | Facing on the pillars of the park's fence | m2 |  | 162.4 |  |
| 9 | Hats on the fence pillars and pillars on the bridge | m |  | 53.00 |  |
| 10 | Mastic on the fence pillars | m3 |  | 3.05 |  |
| 11 | Facing on the beams that connect the pillars in the park | m2 |  | 1.76 |  |
| 12 | Hat on the connecting beams | m |  | 2.34 |  |
| 13 | Mastic on the connecting beams | m3 |  | 0.19 |  |
| 14 | Facing on the baseboard for the railing in the park | m2 |  | 39.32 |  |
| 15 | Mastic on the baseboard for the railing in the park | m3 |  | 0.79 |  |
| 16 | Facing on the supporting wall | m2 |  | 49.35 |  |
| 17 | Hat on the supporting wall | m |  | 32.30 |  |
| 18 | Mastic for the facing of the supporting wall | m3 |  | 1.51 |  |
| 19 | Polymerconcrete wall facing of the inner direct axis board of the bridge | m2 |  | 54 |  |
| 20 | Polymerconcrete wall facing of the outside direct axis of the bridge | m2 |  | 150 |  |
| 21 | Scratch coat under wall facing of the outside board | m3 |  | 2.5 |  |
| 22 | Hat on the inside board of the bridge | m |  | 126.00 |  |
| 23 | Mastic on the inside board of the bridge | m3 |  | 1.30 |  |
| 24 | Fencing element made of compacted concrete (60/80/85)х25х8cm for the bed in front of the pylon | m |  | 13.90 |  |
| 25 | Fencing element made of compacted concrete 0/85)х25х10cm for the bed in front of the pylon | m |  | 13.90 |  |
| 26 | Fencing wall behind the pylon | m2 |  | 12.74 |  |
| 27 | Mastic wall behind the pylon | m3 |  | 0.27 |  |
| 28 | Hat on the wall behind the pylon | m |  | 2.00 |  |
|  | PAVEMENTS ON THE BRIDGE |  |  |  |  |
| 29 | Adjusting cement mastic 2cm | m3 |  | 13.60 |  |
| 30 | Hydroinsulation 1 layer (net area without overlapping) | m2 |  | 458.00 |  |
| 31 | Reinforced cement mastic 4 cm (reinforcement according to the constructive project) | m3 |  | 21.20 |  |
| 32 | Scratch coat 3 cm | m3 |  | 15.90 |  |
| 33 | Bitumen gap on the perimeter | m |  | 132.00 |  |
|  | PAVEMENTS IN THE PARK |  |  |  |  |
| 34 | Scratch coat 3 cm | m3 |  | 29.00 |  |
| 35 | Reinforced concrete pavement 12 cm | m3 |  | 150.00 |  |
| 36 | Polythene 1 layer (without overlapping) | m2 |  | 1250.00 |  |
| 37 | Stamped gravel 25cm | m3 |  | 312.50 |  |
| 38 | Geotextile (without overlapping) | m2 |  | 2040.00 |  |
| 39 | Bitumen gap 2cm on the perimeter | m |  | 245.00 |  |
| 40 | Deformation gap 2 cm (bitumen + stone flour) | m |  | 390.00 |  |
|  | COVERINGS ON THE BRIDGE AND THE PARK |  |  |  |  |
| 41 | Paving blocks made of compacted concrete/polymerconcrete 5 cm | m2 |  | 677.33 |  |
| 42 | Slabs made of compacted concrete/polymerconcrete 40х40х5cm | m2 |  | 530.00 |  |
| 43 | Leading slabs made of compacted concrete/polymerconcrete (60,80,85)х25х5cm | m2 |  | 353.28 |  |
| 44 | Slabs made of compacted concrete/polymerconcrete 20х20х5 cm | m2 |  | 35.72 |  |
| 45 | Slabs made of compacted concrete/polymerconcrete 40х20х5cm | m2 |  | 50.72 |  |
| 46 | Slabs made of compacted concrete/polymerconcrete 50х20х5 cm | m2 |  | 60.40 |  |
| 47 | Slabs made of compacted concrete/polymerconcrete 40х40х5 cm for the foundation of the pylon | m2 |  | 13.90 |  |
|  | PAVEMENTS ON THE SIDEWALK OF THE ENTRANCE |  |  |  |  |
| 48 | Paving blocks made of compacted concrete/polymerconcrete 5 cm | m2 |  | 41.00 |  |
| 49 | Leading slabs made of compacted concrete/polymerconcrete (60,80,85)х25х5cm | m2 |  | 14.80 |  |
| 50 | Scratch coat 3 cm | m3 |  | 1.74 |  |
| 51 | Reinforced concrete pavement 12 cm | m3 |  | 7.11 |  |
| 52 | Polythene 1 layer (without overlapping) | m2 |  | 101.44 |  |
| 53 | Stamped gravel 25cm | m3 |  | 24.50 |  |
| 54 | Geotextile (without overlapping) | m2 |  | 141.20 |  |
| 55 | Bitumen gap | m3 |  | 0.14 |  |
| 56 | Curb | m |  | 16.00 |  |
| 57 | Deformation gap 2 cm (bitumen + stone flour) | m |  | 33.80 |  |
|  | SPECIAL METAL ELEMENTS |  |  |  |  |
| 58 | Delivery and installation of double metal door | piece |  | 1.00 |  |
| 59 | Delivery and installation of single metal doors | piece |  | 2.00 |  |
| 60 | Delivery and installation of a pylon with height 15m | piece |  | 1.00 |  |
|  | HARDWARE WORKS |  |  |  |  |
| 61 | Metal railing with height 101cm along the bridge | m2 |  | 160 |  |
| 62 | Laminated iron folded in a form, painted in blacklead (hides the pipes under the bridge) | m2 |  | 166 |  |
| 63 | Metal railing with height 70cm in the park | m2 |  | 71 |  |
| 64 | Metal railing with height 175cm | piece |  | 2 |  |
|  | ARCHITECTURAL ELEMENTS FOR LUMINAIRES |  |  |  |  |
| 65 | Delivery and installation of luminaires on the bridge | piece |  | 13.00 |  |
| 66 | Delivery and installation of luminaires with four lamps | piece |  | 4.00 |  |
| 67 | Decorative element for installation the luminaires with four lamps | piece |  | 4.00 |  |
|  | NURSERY GARDENS NEXT TO THE TREES |  |  |  |  |
| 68 | Paving blocks made of polymerconcrete 5 cm | m2 |  | 3.24 |  |
| 69 | Mulch | m2 |  | 3.50 |  |
| 70 | Fencing element made of compacted concrete/polymerconcrete (60/80/85)х25х8cm | m |  | 12.00 |  |
| 71 | Fencing element made of compacted concrete/polymerconcrete (60/80/85)х25х10 cm | m |  | 12.00 |  |
|  | FURNISHING |  |  |  |  |
| 72 | Mobile benches with backrest | piece |  | 24.00 |  |
| 73 | Benches for composition 2 | piece |  | 8.00 |  |
| 74 | Decorative corner element for the benches in compositions | piece |  | 8.00 |  |
| 75 | Waste bin | piece |  | 22.00 |  |
|  | OTHERS |  |  |  |  |
| 76 | Glue mixture for the fencing elements and the slabs | m3 |  | 17.6 |  |
| 77 | Gap mixture | m3 |  | 1.58 |  |
| 78 | Mural for the wall behind the pylon | piece |  | 1.00 |  |
| 79 | Scaffolding | м |  | 188.00 |  |
| II | PART: CONSTRUCTIONS |  |  |  |  |
|  | CONCRETE WORKS | В10 |  | 5.30 |  |
|  |  | В25 |  | 248.40 |  |
| 1 | Concrete for concrete fields | m3 |  | 118.10 |  |
| 2 | Exposed concrete for foundations and edge beams | m3 |  | 5.30 |  |
| 3 | Concrete for columns, foundations and edge beams | m3 |  | 102.40 |  |
| 4 | Concrete for existing concrete wall | m3 |  | 15.00 |  |
| 5 | Concrete for wall 1 - 19 pieces. | m3 |  | 1.80 |  |
| 6 | Concrete for wall 2 - 1 piece | m3 |  | 0.70 |  |
| 7 | Concrete for wall 3 - 11 pieces | m3 |  | 2.20 |  |
| 8 | Concrete for boards | m3 |  | 8.20 |  |
|  | SHUTTERING WORKS |  |  |  |  |
| 9 | Shuttering for exposed concrete for foundations and edge beams | m2 |  | 20.50 |  |
| 10 | Shuttering for concrete for columns, foundations and edge beams | m2 |  | 435.61 |  |
| 11 | Shuttering for concrete for existing concrete wall | m2 |  | 49.89 |  |
| 12 | Shuttering for wall 1 - 19 pieces | m2 |  | 52.72 |  |
| 13 | Shuttering for wall 2 - 1 piece | m2 |  | 8.55 |  |
| 14 | Shuttering for wall 3 - 11 pieces | m2 |  | 49.61 |  |
| 15 | Shuttering for boards | m2 |  | 100.90 |  |
|  | REINFORCEMENT WORKS |  |  |  |  |
| 16 | Reinforcement for single net N8/15cm for concrete fields | kg |  | 5211.40 |  |
| 17 | Reinforcement for foundations and edge beams | kg |  | 2091.00 |  |
| 18 | Reinforcement for columns | kg |  | 802.00 |  |
| 19 | Reinforcement for existing wall | kg |  | 458.00 |  |
| 20 | Reinforcement for wall 1 - 19 pieces | kg |  | 239.00 |  |
| 21 | Reinforcement for wall 2 - 1 piece | kg |  | 61.00 |  |
| 22 | Reinforcement for wall 3 - 11 pieces | kg |  | 205.00 |  |
| 23 | Reinforcement for boards | kg |  | 1170.00 |  |
|  | STEEL CONSTRUCTION |  |  |  |  |
| 24 | Pylon - 390 kg | piece |  | 1 |  |
|  | OTHER WORKS |  |  |  |  |
| 25 | Gaps in the concrete pavement | m |  | 398.92 |  |
| 26 | Pipe j110mm-L=0,12m in concrete fields | piece |  | 300.00 |  |
|  | RECONSTRUCTION WORKS ON THE BRIDGE'S CONSTRUCTION |  |  |  |  |
| 27 | Clean the reinforcement from the products of corrosion | m2 |  | 405.00 |  |
| 28 | Execution of anti-corrosion defense to the reinforcement and adhesive layer with PAGEL MS02 | m2 |  | 1250.00 |  |
| 29 | Restoration of the integrity of the project section of the stabilizing elements up to 2 cm with PAGEL MS20 | m2 |  | 280.00 |  |
| 30 | Restoration of the integrity of the project section of the stabilizing elements up to 4 cm with PAGEL MS40 | m2 |  | 165.00 |  |
| III | PART: WATER SUPPLY AND SEWERAGE |  |  |  |  |
|  | SEWERAGE |  |  |  |  |
| 1 | Demolition and reconstruction of asphalt pavement | m2 |  | 8.00 |  |
| 2 | Demolition and reconstruction of stone flat sheets pavement | m2 |  | 3.84 |  |
| 3 | Soil ditch - manual | m3 |  | 7.99 |  |
| 4 | Soil ditch - machinery | m3 |  | 31.95 |  |
| 5 | Transport of earth masses to depot | m3 |  | 39.95 |  |
| 6 | Delivery of sand for pad and reverse embankment | m3 |  | 29.04 |  |
| 7 | Delivery of inert materials for reverse embankment | m3 |  | 2.69 |  |
| 8 | Sand pad | m3 |  | 5.00 |  |
| 9 | Reverse embankment of ditch with sand over pipe vertex | m3 |  | 24.05 |  |
| 10 | Filling up narrow ditches with earth masses without ramming | m3 |  | 8.21 |  |
| 11 | Filling up narrow ditches with inert materials without ramming | m3 |  | 2.69 |  |
| 12 | Ditch packing | m3 |  | 34.95 |  |
| 13 | Delivery and installation of a pipe PVC Ф75 with access fitting and all pursuing actions | m |  | 2.00 |  |
| 14 | Delivery and installation of pipe PVC Ф160 SN8 with access fittings and all pursuing actions | m |  | 28.50 |  |
| 15 | Delivery and installation of pipe PVC Ф200 SN8 with access fittings and all pursuing actions | m |  | 33.90 |  |
| 16 | Construction of gully access Ф630 С Н= up to 2m. | piece |  | 3.00 |  |
| 17 | Construction of an opening in reinforced concrete slab Ф75 | piece |  | 2.00 |  |
| 18 | Delivery and installation of rain water furrows, height 60mm, width 147, with ribbed grating with staple, locked, with all pursuing actions | m |  | 9.00 |  |
| 19 | Delivery and installation of street drainage – 30/30 made of phaserconcrete, with anchored steel edge, in a kit with a basket for trash, water valve against odor, cast iron net-shaped grating with an eye 20/30 locked without bolts, class Е600, with construction size: width 300 mm, length 300mm, height 400 mm. Water quantity 5 l/sec. | piece |  | 2.00 |  |
| 20 | Delivery and installation of ready to use concrete furrows for construction of an open-air channel | m |  | 3.50 |  |
| 21 | Grassing of green zones | m2 |  | 38.08 |  |
| IV | PART: ELECTRIC |  |  |  |  |
| 1 | Delivery and installation of lighting board for the bridge, IP-65, installation - standing | piece |  | 1.00 |  |
| 2 | Tracing of cable line | m |  | 80.00 |  |
| 3 | Ditch with filling and ramming /soil of 3rd category/ - 0,8/0,4m | m |  | 80.00 |  |
| 4 | Construction of a well with cover 60/90sm | piece |  | 5.00 |  |
| 5 | Delivery of РVС pipes ф50, installation | m |  | 400.00 |  |
| 6 | Delivery of РVС pipes ф20, installation | m |  | 480.00 |  |
| 7 | Delivery of gas pipe 2", installation | m |  | 160.00 |  |
| 8 | Delivery and drawning in a cable СВТ 5х2,5 mm2 | m |  | 350.00 |  |
| 9 | Delivery and drawning in a cable СВТ 3х1,5 mm2 | m |  | 260.00 |  |
| 10 | Delivery and drawning in a cable СВТ 4х1,5 mm2 | m |  | 120.00 |  |
| 11 | Delivery and drawning in a cable СВТ 4х2,5 mm2 | m |  | 60.00 |  |
| 12 | Delivery and drawning in a cable СВТ 3х2,5 mm2 | m |  | 60.00 |  |
| 13 | Dry partition of cable up to 6mm2 | piece |  | 100.00 |  |
| 14 | Connecting of conductor up to 6mm2 | piece |  | 300.00 |  |
| 15 | Delivery and installation of a projector with МХЛ 50W/220V, IP65, /lighting green wall/ | piece |  | 6.00 |  |
| 16 | Delivery and installation of a projector with 50W/220V, IP65, / lighting coat of arm/ | piece |  | 1.00 |  |
| 17 | Delivery and installation of luminaire with LED lighter to 20W, IP65, installation - standing | piece |  | 17.00 |  |
| 18 | Delivery and installation of luminaire with LED lighter to 20W, IP65, installation - ceiling, under bridge | piece |  | 56.00 |  |
| 19 | Delivery and installation of luminaire, bracket type with LED lighter to 20W, IP65, installation - wall | piece |  | 16.00 |  |
| 20 | Delivery and installation of luminaire, type projector with LED lighter to 50W, IP65, installation - in the pavement | piece |  | 3.00 |  |
| 21 | Delivery of luminaire with 1 piece МХЛ lighter 50W on a pillar H=3,0m, with built in defence box type "К2-А" with one piece automatic device 10А, monophase | piece |  | 13.00 |  |
| 22 | Delivery of luminaire with 1 piece КЛЛ lighter 18W , for building up in a railing, IP65 | piece |  | 26.00 |  |
| 23 | Delivery and installation of an earthing bar in concrete | m |  | 150.00 |  |
| 24 | Construction of earthing with earthing element made of steel zink-coated profiles 63/63/6mm, 1,5m - depth 2,5m | piece |  | 5.00 |  |
| 25 | Construction of earthing heel with a bar 40/4mm with length to 1.0м | piece |  | 14.00 |  |
| 26 | Connection of earthing bar with bolt connection of the board | piece |  | 14.00 |  |
| 27 | El. Measuring | h |  | 15.00 |  |
| V | PART: PLANTING |  |  |  |  |
|  | Broad-leaved trees |  |  |  |  |
| 1 | Acer platanoides 'Royal Red' - size 14/16 | piece |  | 2.00 |  |
|  | Coniferous bushes |  |  |  |  |
| 2 | Juniperus procumbens 'Nana' - size co 1.5 l | piece |  | 60.00 |  |
|  | Broad-leaved bushes |  |  |  |  |
| 3 | Lonicera nitida 'Elegant' - size co 1.5 l | piece |  | 20 |  |
| 4 | Santolina chamacyparissus - size co 1.5 l | piece |  | 175 |  |
|  | Seasonal flowers |  |  |  |  |
| 5 | Ageratum houstonianim | piece |  | 270.00 |  |
| 6 | Tagettes | piece |  | 240.00 |  |
|  | Vegetation in cachepots |  |  |  |  |
| 7 | Prunus laurocerassus 'Otto Lyunken' - size co 1.5 l | piece |  | 32.00 |  |
|  | DESCRIPTION OF RECONSTRUCTION WORKS |  |  |  |  |
| 8 | Filling up with humus layer /with turning coefficient 1,29% and depth 40cm. / | m3 |  | 23.00 |  |
| 9 | Preparation of the terrain for grassing | m2 |  | 47.50 |  |
| 10 | Filling the cachepots with fertilizer soil | m3 |  | 1.40 |  |
| 11 | Filling in with felt in order to drain the cachepots | m3 |  | 0.20 |  |
| 12 | Laying of geotextile for cachepots | m2 |  | 12.00 |  |
| 13 | Filling mulch 10 cm thick. around the trunk of trees | m3 |  | 0.50 |  |
| VI | PART: VERTICAL PLANNING |  |  |  |  |
|  | DEMOLITION |  |  |  |  |
| 1 | Demolition of existing asphalt pavement including loading, transportation, unloading at a depot pointed by the Investor and all pursuing expenses - 1593 м2 | m3 |  | 95.60 |  |
| 2 | Demolition of existing basalt plates, including loading, transportation, unloading at a depot pointed by the Investor and all pursuing expenses. | m2 |  | 30.0 |  |
| 3 | Demolition of an existing massive stone wall 0,45х0,65х40,0 and stone nursery gardens 0,70х0,3х62,7 including loading, transportation, unloading at a depot pointed by the Investor and all pursuing expenses | m3 |  | 24.9 |  |
|  | EARTH WORKS |  |  |  |  |
| 4 | Machinery digging of useless waste fraction ballast, layer on working altitude from the Cartogram of earth masses and Details for the pavements | m3 |  | 352.00 |  |
| 5 | Loading and transportation to a depot for building waste /Q turning = 1.15 / appointed by the Investor and all pursuing expenses. | m3 |  | 305.00 |  |
| 6 | Packing the earth base with steam roller - /Е = 30 МРа/ | m2 |  | 980.00 |  |
| VII | PART: CO-FINANCING |  |  |  |  |
| 1 | Expansion joints | nmbr |  | 2 |  |
|  | Total lump-sum price VAT excluded |  |  |  |  |
|  | VAT |  |  |  |  |
|  | Total lump-sum price VAT included |  |  |  |  |

Note: For all items for delivery and installation of luminaires, the price must include all necessary costs for delivery and all necessary elements for the installation.